INFORMATION RELATIVE TO

THE DRAFT TITLE V OPERATING PERMIT

(March 8, 2013)

FOR:

Southern Natural Gas Company, LLC

Muldon Storage Facility

10156 Old Magnolia Highway

Prairie, Mississippi (Monroe Co.)

FACILITY DESCRIPTION

Southern Natural Gas Company, LLC –Muldon Storage Facility, is an existing natural gas transmission and storage facility (SIC 4922) located near Prairie, Mississippi (Monroe County).  The facility is applying for renewal of the current Title V Operating Permit.  The facility consists of the following significant sources:

| Emission Point | Description |
| --- | --- |
| AA-001 | Clark 7,250 HP natural gas-fired two stroke lean burn internal combustion engine (Compressor Engine Model No. TCVD-16, ref. no. 001C-001) |
| AA-002 | Clark 7,250 HP natural gas-fired two stroke lean burn internal combustion engine (Compressor Engine Model No. TCVD-16, ref. no. 001C-002) |
| AA-003 | Clark 7,250 HP natural gas-fired two stroke lean burn internal combustion engine (Compressor Engine Model No. TCVD-16, ref. no. 001C-003) |
| AA-004 | Caterpillar 930 HP (6.5 MMBtu/hr) natural gas-fired four stroke rich burn emergency generator (Model No. G-399, ref. no. 005G-001) |
| AA-005 | Caterpillar 930 HP (6.5 MMBtu/hr) natural gas-fired four stroke rich burn emergency generator (Model No. G-399, ref. no. 005G-002) |
| AA-007 | Cleaver Brooks 1.5 MMBTU/hr natural gas-fired boiler (ref. no. 005B-001) |
| AA-008 | National 7.38 MMBTU/hr natural gas-fired withdrawal heater (Model No. B-22416, ref. no. 002H-001) |
| AA-009 | National 7.38 MMBTU/hr natural gas-fired withdrawal heater (Model No. B-22416, ref. no. 002H-002) |
| AA-010 | National 7.38 MMBTU/hr natural gas-fired withdrawal heater (Model No. B-22416, ref. no. 002H-003) |
| AA-011 | National 15.38 MMBTU/hr natural gas-fired withdrawal heater ( ref. no. 002H-004) |
| AA-012 | National 15.38 MMBTU/hr natural gas-fired withdrawal heater ( ref. no. 002H-005) |
| AA-013 | Smalling 15.29 MMBTU/hr natural gas-fired regeneration heater ( ref. no. 002H-006) |
| AA-014 | Natco 15.0 MMBTU/hr natural gas-fired regeneration heater ( ref. no. 002H-007) |
| AA-015 | Natco 15.0 MMBTU/hr natural gas-fired regeneration heater ( ref. no. 002H-008) |
| AA-016 | Natco 21.76 MMBTU/hr natural gas-fired regeneration heater ( ref. no. 002H-009) |
| AA-017 | 16,800 gallon natural gas condensate tank with a fixed roof (ref. no. 003T-019) |
| AA-018 | 16,800 gallon natural gas condensate tank with a fixed roof (ref. no. 003T-020) |
| AA-019 | 16,800 gallon natural gas condensate tank with a fixed roof (ref. no. 003T-021) |
| AA-020 | 16,800 gallon natural gas condensate tank with a fixed roof (ref. no. 003T-022) |
| AA-021 | 16,800 gallon natural gas condensate tank with a fixed roof (ref. no. 003T-023) |
| AA-022 | 16,800 gallon natural gas condensate tank with a fixed roof (ref. no. 003T-024) |
| AA-023 | Silvalls 11.3 MMBTU/hr natural gas-fired withdrawal heater |

The permit renewal is not believed to trigger or require a Prevention of Significant Deterioration (PSD) review.  The facility is a natural gas compressor station with criteria emissions that exceed 250 tons per year.  Since emissions exceed the PSD threshold, the facility is classified as an existing major PSD source.  Therefore, any modifications as defined by the PSD regulations require a PSD review.  No modifications to the facility have occurred that are believed to require a PSD review.

TITLE V PROGRAM APPLICABILITY BASIS

The facility’s potentialtoemit exceeds the Title V threshold limits of 100 tons per year (TPY) for Nitrogen Oxides (NOX), Carbon Monoxide (CO), and Volatile Organic Compounds (VOC) and in excess of the 10 TPY threshold for an individual Hazardous Air Pollutant (HAP) – Formaldehyde and 25 TPY of total HAPs. Therefore, this facility is a major source for Title V and HAP emissions.

LEGAL AND FACTUAL BASIS FOR DRAFT PERMIT CONDITIONS

The State and Federally-enforceable conditions of Title V Operating Permits are based upon the requirements of the State of Mississippi Air Emissions Operating Permit Regulations for the Purposes of Title V of the Federal Clean Air Act (APC-S-6), and applicable requirements effective upon the date of permit issuance.  Applicable requirement means all of the following as they apply to emissions units in a Title V source:

1.	any standard or other requirement set forth in the State Implementation Plan (SIP) approved or promulgated by EPA through rulemaking under Title I of the Federal Clean Air Act (Federal Act) including the following:

a.	most of the State of Mississippi Air Emission Regulations for the Prevention, Abatement, and Control of Air Contaminants (APC-S-1)

b.	the State of Mississippi Regulations for the Prevention of Air Pollution Emergency Episodes (APC-S-3),

c.	the State of Mississippi Regulations for the Prevention of Significant Deterioration of Air Quality (APC-S-5), and 40 CFR Part 52.21 by reference, and

d.	the provisions of the State of Mississippi Permit Regulations for the Construction and/or Operation of Air Emissions Equipment (APC-S-2), relating to construction permits and synthetic minor operating permits;

2.	any term or condition of any construction permits issued pursuant to Mississippi regulations approved or promulgated through rulemaking under Title I;

3.	any standard or other requirement under Section 111 of the Federal Act, including Section 111(d) which includes Title 40, Part 60 of the Code of Federal Regulations (40 CFR Part 60) and relevant sections of APC-S-1;

4.	any standard or other requirement under Section 112 of the Federal Act, including relevant sections of APC-S-1 and 40 CFR Parts 61, 63, and 68;

5.	any standard or other requirement of the acid rain program under Title IV of the Federal Act or the regulations promulgated thereunder, including the State of Mississippi Acid Rain Program Permit Regulations for Purposes of Title IV of the Federal Clean Air Act (APC-S-7)  adopted November 17, 1994, and 40 CFR Parts 72, 73, 75, 77, and 78;

6.	any requirements established pursuant to Section 504(b) or Section 114(a)(3) of the Federal Act;

7.	any standard or other requirement governing solid waste incineration under Section 129 of the Federal Act;

8.	any standard or other requirement for consumer and commercial products under Section 183(e) of the Federal Act;

9.	any standard or other requirement for tank vessels under Section 183(f) of the Federal Act;

10.	any standard or other requirement of the program to control air pollution from outer continental shelf sources under Section 328 of the Federal Act;

11.	any standard or other requirement of the regulations promulgated to protect stratospheric ozone under Title VI of the Federal Act;

12.	any national ambient air quality standard or increment or visibility requirement under part C of Title I of the Federal Act.

Each State and Federally-enforceable condition of the draft Title V Operating Permit references the specific relevant requirements of APC-S-6 or the applicable requirement upon which it is based. Any condition of the draft Title V permit that is enforceable by the State but is not Federally-enforceable is identified in the draft Title V permit as such.

CAM APPLICABILITY

40 CFR 64 specifies the requirements for Compliance Assurance Monitoring (CAM).  The general applicability is in 40 CFR 64.2 and requires any Part 70 or Part 71 source to comply with the CAM requirements if the following criteria are met:

- The unit is subject to an emission limitation or standard for a regulated air pollutant other than exemptions under 40 CFR 64.2(b)(1), and

- The unit uses a control device to meet the standard, and

- The unit has pre-control emissions exceeding major source thresholds.

The facility has equipment with pre-control emissions exceeding the major source thresholds but does not use control devices to comply with an emission limit or standard.  Therefore, CAM does not apply.

NSPS APPLICABILITY

Although the facility operates stationary spark ignition (SI) internal combustion engines, which are potentially subject to 40 CFR 60, Subpart JJJJ, the units are considered existing units and were manufactured prior to the applicability date.  Therefore, Subpart JJJJ is not applicable to these units.

Emission Point AA-023 is subject to the New Source Performance Standards (NSPS) for Small Industrial-Commercial-Institutional Steam Generating Units, 40 CFR Part 60, Subpart Dc.  This is a new natural gas-fired unit and is required to meet the applicable requirements of this standard upon startup and the General Provisions, 40 CFR Part 63, Subpart A.  (Ref.:  40 CFR 60 Subpart Dc, 60.40c)

The facility is not believed to be subject to any other NSPS.

MACT APPLICABILITY

This facility is a major source for HAP emissions (Total HAPs > 25 TPY).

Emission Points AA-001 through AA-003 are subject to the National Emission Standards for Hazardous Air Pollutants (NESHAP) for Stationary Reciprocating Internal Combustion Engines (RICE), 40 CFR Part 63, Subpart ZZZZ.  These units are existing stationary RICE’s that are 2-stroke lean burn units and are not required to meet the requirements of this standard or the General Provisions, 40 CFR Part 63, Subpart A.  (Ref.:  40 CFR 63.6585 & 63.6590(b)(3))

Emission Points AA-004 and AA-005 are subject to the NESHAP for Stationary RICE, 40 CFR Part 63, Subpart ZZZZ.  The units are existing stationary RICE that are 4-stroke rich burn units and are also emergency use units that are not required to meet the requirements of this standard, except for 40 CFR 63.6640(f)(2), or General Provisions, 40 CFR Part 63, Subpart A.(Ref.: 40 CFR 63.6585, 63.6590(b)(3), & 63.6640(f)(2))

Emission Points AA-007 through AA-016 are subject to the National Emission Standards for Hazardous Air Pollutants (NESHAP) for Major Sources: Industrial, Commercial, and Institutional Boilers and Process Heaters, 40 CFR Part 63, Subpart DDDDD.  These units are existing natural gas-fired units and are required to meet the applicable requirements of this standard by March 21, 2014 and the General Provisions, 40 CFR Part 63, Subpart A.  (Ref.:  40 CFR 63 Subpart DDDDD, 63.7480, 63.7485, 63.7490(a)(1), and 63.7495(b))

Emission Point AA-023 is subject to the NESHAP for Major Sources: Industrial, Commercial, and Institutional Boilers and Process Heaters, 40 CFR Part 63, Subpart DDDDD.  This is a new natural gas-fired unit and is required to meet the applicable requirements of this standard upon startup and the General Provisions, 40 CFR Part 63, Subpart A.  (Ref.:  40 CFR 63 Subpart DDDDD, 63.7480, 63.7485, 63.7490(a)(2), and 63.7495(a))

40 CFR 63, Subparts HH (Oil and Gas Production) and HHH (Natural Gas Transmission and Storage) are potentially applicable to site operations and equipment since the site is a major source of HAPs and involves natural gas transmission.  However, Subpart HH does not apply because the facility does not produce oil or natural gas.  With respect to Subpart HHH, the site is not a natural gas storage facility (transferred only).  The site is also not considered a transmission facility, since 63.1270(b) & (c) specifies that “the affected source is each glycol dehydration unit”.  The site does not utilize glycol dehydrators, and therefore is not subject to Subpart HHH.

The facility is not believed to be subject to any other NESHAP.

SPECIFIC APPLICABLE REQUIREMENTS

| EMISSION LIMITS |  |  |  |
| --- | --- | --- | --- |
| Emission Point No. | Pollutant/Parameter | Draft Permit Emission Limits | Monitoring Requirements |
| AA-001 through AA-003, AA-011 through AA-016, & AA-023 | PM/PM10 | E = 0.8808*(I)-0.1667 | Monitor and record monthly the type and quantity of natural gas combusted, including monthly gas analysis with sulfur content and heating value. |
| AA-004 through AA-010 | PM/PM10 | 0.6 lb/MMBTU |  |
| AA-001 through AA-016& AA-023 | SO2 | 4.8 lbs/MMBTU |  |
| AA-001 through AA-016 | Fuel | Limited to pipeline quality natural gas only. |  |
| AA-001 through AA-003 | HAP | MACT applicability only and does not have to meet the requirements of this standard. | N/A |
| AA-004 and AA-005 | HAP | MACT applicability only and does not have to meet the requirements of this standard except for 63.6640(f)(2) as provided for in definition for emergency generator. | Monitor and record all periods of operation on a monthly basis and identify hours of operation used for routine testing and maintenance and hours of operation during non-emergency use. |
| AA-004 and AA-005 | Operation | Limit of 500 hours per year of operation. |  |
| AA-007 through AA-016 | HAP | 40 CFR 63, Subpart DDDDD applicability requirements for existing natural gas-fired units, which must comply by March 21, 2014. | Conduct one-time energy assessment and (annual, biennial, or 5-yr) tune up. |
| AA-023 | HAP | 40 CFR 63, Subpart DDDDD applicability requirements for new natural gas-fired units, which must comply upon startup. | Conduct one-time energy assessment and annual tune up. |
| AA-023 | Fuel Usage | Limited to natural gas only. | Record and maintain records of the amount of each fuel combusted on a monthly basis. |

OTHER LIMITS:  N/A