# PERMIT RATIONALE FOR REISSUANCE

Denbury Onshore LLC, Olive Field EOR Facility

April 22, 2026

- FACILITY INFORMATION

Facility Name:  Denbury Onshore LLC, Olive Field EOR Facility

Facility Address:  7969 Robert Jones Road, Summit, MS  39666

Facility County:  Amite County

EPA ID.:  110020065114

Water NPDES Permit No.:  MS0059587

Standard Industrial Classification (SIC) Code:  1311 (Crude Petroleum and Natural Gas)

Permit Writer:  Deanna Rush

- NATURE OF BUSINESS

Denbury Onshore LLC, Olive Field EOR Facility is an enhanced oil recovery facility. They inject carbon dioxide into production wells to extract oil.  Carbon dioxide recycles back to the formation via injection wells. Oil is sold via tanker truck and produced water is disposed of via Class II Injection Well. This facility is not currently operating, but Denbury requested that the permit remain open should operations restart. The facility is a minor NPDES source

PERMIT MODIFICATION

A cooling tower has functioned to cool and compress carbon dioxide for re-injection into production wells. Denbury no longer uses the cooling water tower and has requested it’s removal from the permit.  A site visit by the permit writer verified that cooling tower discharge pipes have been disconnected from Outfall 001.  Previously this outfall discharged cooling tower blowdown to an unnamed ditch of Tangipahoa River.

- EFFLUENT AND RECEIVING STREAM FLOW DATA

Outfall 001 - Cooling Tower Blowdown (Removed from Permit)

Outfall 002 - discharges stormwater at a daily maximum flowrate of 0.1347 MGD from the Oil Storage Secondary Containment Area to an unnamed ditch (7Q10=0) then to Tangipahoa River in South Independent Streams Basin. The amount of effluent discharged is rainfall dependent.

Outfall 003 - discharges stormwater at a daily maximum flowrate of 0.7049 MGD from the East Yard Containment Area to an unnamed ditch (7Q10=0) of Tangipahoa River in South Independent Streams Basin. The amount of effluent discharged is rainfall dependent.

Outfall 004 - discharges stormwater runoff at a daily maximum flowrate of 0.8396 MGD from the West Yard Containment Area to an unnamed ditch (7Q10=0) of Tangipahoa River in South Independent Streams Basin. The amount of effluent discharged is rainfall dependent.

- IMPAIRED WATER BODIES 303(d) and TOTAL MAXIMUM DAILY LOAD (TMDL)

Tangipahoa River is not listed on the 2024 (adopted) or 2026 (draft) List of Impaired Water Bodies (303 (d) List). There is a Fecal Coliform TMDL for Tangipahoa River, South Independent Streams Basin, Pike and Amite Counties, December 1999 – ID# 1181; however, it does not apply to effluent from this facility.

- TYPE OF WASTEWATER TREATMENT:

Outfall 002 – Sedimentation in Containment Area

Outfall 003 – Sedimentation in Containment Area

Outfall 004 – Sedimentation in Containment Area

- EPA APPLICABLE CATEGORICAL GUIDELINES

Not Applicable.

DATA FROM APPLICATION FORM (2C)

| Parameter | Outfall 002 Maximum Daily Discharge Concentration | Outfall 003 Maximum Daily Discharge Concentration | Outfall 004 Maximum Daily Discharge Concentration |
| --- | --- | --- | --- |
| Biochemical Oxygen Demand (BOD 5-Day) | <6 mg/L | <6 mg/L | <6 mg/L |
| Chemical Oxygen Demand (COD) | 27 mg/L | 30 mg/L | 16 mg/L |
| Total Organic Carbon | 10.3 mg/L | 10.3 mg/L | 8.0 mg/l |
| Total Suspended Solids | 4 mg/L | 4 mg/L | 6 mg/L |
| Ammonia (as N) | <0.1 | <0.1 | <0.1 |
| 1 Flow | 0.1347 MGD | 0.7049 MGD | 0.8396 MGD |
| Temperature | Ambient | Ambient | Ambient |
| pH (minimum) | 6.3 | 6.0 | 6.4 |
| pH (maximum) | 8.5 | 8.3 | 8.2 |
| Oil and grease | < 1.6 mg/L | Believed Absent | Believed Absent |

1Flow values were estimated.

#### VIII. 	WATER QUALITY LIMITATIONS BASED ON WASTELOAD ALLOCATION

Not Applicable

- CATEGORICAL GUIDELINE LIMITATIONS CALCULATIONS

Not Applicable

TOXICITY SCREENING

Not Applicable

PROPOSED FINAL LIMITATIONS (Outfall 001 has been removed)

- Average Permit Limitations

| Outfall | Parameter | Water Quality Limitation | Previous Permit Limitation | Proposed Permit Limitation | Monitoring Frequency | Sample Type | DMR Reporting Frequency | 1Basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 002 | Flow (MGD) | --- | Report | Report | Twice per Month | Estimate | Monthly | PPL |
| 002 | Oil and grease | --- | 10 | 10 | Monthly | Grab Sampling | Monthly | PPL |
| 002 | pH (min) | 6.0 SU | 6.0 SU | 6.0 SU | Twice per month | Grab Sampling | Monthly | MSWQS |
| 003 | Flow (MGD) | --- | Report | Report | Twice per month | Estimate | Monthly | PPL |
| 003 | pH (min) | 6.0 SU | 6.0 SU | 6.0 SU | Twice per month | Grab Sampling | Monthly | MSWQS |
| 004 | Flow (MGD) | --- | Report | Report | Twice per month | Estimate | Monthly | PPL |
| 004 | pH (min) | 6.0 SU | 6.0 SU | 6.0 SU | Twice per month | Grab Sampling | Monthly | MSWQS |

- Maximum Permit Limitations

| Outfall | Parameter | Water Quality Limitation | Previous Permit Limitation | Proposed Permit Limitation | Monitoring Frequency | Sample Type | DMR Reporting Frequency | 1Basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 002 | Flow (MGD) | --- | Report | Report | Twice per Month | Estimate | Monthly | PPL |
| 002 | Oil and grease | --- | 15 | 15 | Monthly | Grab Sampling | Monthly | PPL |
| 002 | pH (max) | 9.0 SU | 9.0 SU | 9.0 SU | Twice per month | Grab Sampling | Monthly | MSWQS |
| 003 | Flow (MGD) | --- | Report | Report | Twice per month | Estimate | Monthly | PPL |
| 003 | pH (max) | 9.0 SU | 9.0 SU | 9.0 SU | Twice per month | Grab Sampling | Monthly | MSWQS |
| 004 | Flow (MGD) | --- | Report | Report | Twice per month | Estimate | Monthly | PPL |
| 004 | pH (max) | 9.0 SU | 9.0 SU | 9.0 SU | Twice per month | Grab Sampling | Monthly | MSWQS |

1Basis: MSWQS = Mississippi Water Quality Standards, PPL=Previous Permit Limits.